METHOD STATEMENT TEMPLATE AND GUIDELINE

Document Reference Number: EM-7500-M-6051-RA

Location: YASI S/S to BHYG S/S

Work Scope: Installing Route Markers and Crash Poles

Client Contract No: N 19084

Contractor is responsible to prepare method statement for each work activities with compliance to RFC drawings, standard drawings, project specification, prevailing international standards and other relevant approvals and submit it for consultant/TAQA review and approval.

This method statement guideline is prepared in good faith in line with standard engineering practices, to guide, facilitate contractors and standardize the procedural submissions. However, it does not relieve the Contractor from their contractual obligations.

Any discrepancy between this information and actual site conditions shall be verified for its correctness and suitability, as per applicable TAQA procedures and local authorities. The Contractor shall verify and confirm submissions case to case, as per applicable law and standards.

| DOCUMENT REVIEW & AUTHORISATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor |  | APPROVAL |  |  |  |  |  |
|  |  | Contractor Project Manager | Sub-Contractor Project Manager | Site Manager | Duty Holder/ HSE Manager | Client/Client Representative |  |
| Name |  | Mr. R Sasitharan | Yousuf Suleiman |  |  |  |  |
| Signature /Date |  |  |  |  |  |  |  |
| 1 | Does this document identify high risk activities? Yes / No If answer is “Yes” then risk assessment for each individual high-risk element of work shall be included. |  |  |  |  |  |  |
| 2 | Does the work scope include temporary works? Yes / No If answer to above question is “Yes” then a separate risk assessment/method statement shall be submitted for temporary works in addition to the method statement risk assessment for the main works |  |  |  |  |  |  |

|  | DISTRIBUTION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ORGANISATION NAME |  | NAME | POSITION | SIGNATURE | DATE |  |

Sub-Contractor  EPC Contractor  (OE) PM Consultant appointed by the Client  Client

|  | REVISION RECORD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Revision |  |  | Date |  |  | Comments / Changes |  |
| 0 |  |  | 01/05/2024 |  |  | First issue |  |  |
|  | Field Change Notices |  |  |  |  |  |  |  |

Contents

Contents

Abbreviations Used	3

1. Purpose and Scope of Works	4

2. Programme and Constraints	5

3. Surveys and Engineering Design	6

4. 4.Specifications & Standards	8

5. 5. Permits and Permissions	9

6. Specific Legal Compliance	10

7. Resources & Competency	11

8. Communication	13

9. Supervision and Responsibilities	14

10. Health Impacts	15

11. Quality Control	16

12. Environmental Impacts	18

13. Methodology and Sequence of Works	20

14. Emergency Procedures	23

15. Appendices	24

16. Risk Assessments	25

17. Sign Off Sheet	26

# Abbreviations Used

| Abbreviations Used | Definition |
| --- | --- |
| ILO R164 | Occupational Safety and Heath Recommendation |
| ILO R175 | Safety and Heath in Construction |
| PPE | Personal protective equipment |
| COSHH | Control of Substances Hazardous to Health |

# Purpose and Scope of Works

This Method Statement covers all works associated with the followings:

- Installing Route Markers and Crash Poles

Contractor to include their details below:

Purpose

This method statement describes the procedure, responsibilities, materials, tools, and safety measures to be followed for the installation of route markers and crash poles along the 132kV underground cable route, in compliance with the project specifications, drawings, and TRANSCO standards.

Scope

This procedure covers: - Installation of 132kV route markers along the cable route at specified intervals and at each change in direction, joint bay, and road crossing. - Installation of crash poles to protect route markers and joint bay markers in road shoulders or exposed areas. - Ensuring all markers and poles are properly aligned, levelled, and concreted as per approved construction drawings.

# Programme and Constraints

| Duration of Works |  |  |  |  |
| --- | --- | --- | --- | --- |
| Anticipated start date: 13 Feb 2026 |  | Anticipated finish date: 07 Aug 2026 |  | Duration: 6 Months |
| Working Hours |  |  |  |  |
| Weekday | Start time: 7.00 a.m |  | Finish time: 5.00 p.m |  |
| Weekend | Start time: |  | Finish time: |  |
| Generic Constraints |  |  |  |  |
| Environmental: Spilling of fuel or hydraulic oil on site |  |  |  |  |
| Equipment/Plant/Machinery: Excavator |  |  |  |  |
| Site Specific Constraints (to be identified by the Contractor) |  |  |  |  |
| Environmental: Identify Potential Sources of Waste: Wood, paper. Waste Segregation and Storage: Establish clearly labelled waste collection bins for different types of waste, such as recyclables, hazardous materials, and general waste. Ensure proper storage of hazardous materials in designated areas with appropriate containment measures to prevent spills or leaks. Waste Disposal and Recycling: Identify local regulations and guidelines for the disposal of different types of waste, including hazardous materials. Contract with licensed waste management companies for the proper disposal of hazardous materials and recycling of recyclable materials. Implement procedures for the safe transportation of hazardous materials to disposal facilities. Monitoring and Evaluation: Regularly monitor waste generation and disposal practices to identify areas for improvement. Conduct periodic audits to ensure compliance with waste management procedures and regulations. Dust can be produced in dry condition; as a result dust mask may be required. If required, water spray will be used to suppress dust. |  |  |  |  |
| Equipment/Plant/Machinery: |  |  |  |  |
| Existing Infrastructure: The presence of any existing structures or underground elements |  |  |  |  |
| Utilities: Before starting excavation, refer the GIS drawings, as built drawings, inspect each trial pit location and identify existing underground utilities using a valid calibrated cable detector and GPR detection methods and maintain a proper record to avoid damaging live utilities |  |  |  |  |
| Access/Egress etc.: Only operatives with valid third-party training certificates will be allowed on site. The site will have designated walkways/walking routes for pedestrians and roads for vehicles, with a speed limit of 20 km/hr. Parking will be available on site for personnel. The work area will be clearly demarcated to restrict access and mitigate unauthorized entry. |  |  |  |  |

NOTE:

- Working days/times subject to TAQA/Local authority approval

- Night shift working hours subject to TAQA/Local authority approval

- Working near live TAQA asset subject to TAQA systems safety rules (SSR)

# Surveys and Engineering Design

The Contractor shall provide necessary data survey support from site in order to identify the risk implication based on actual site conditions such as congested areas, underground services, etc.

The following factors can influence the selection of construction method and equipment:

- Terrain

- Soil condition

- Environmental and public constraints

- Accessibility to the location and limitation of the equipment

- Flooding

The table below shall be completed by the Contractor.

| Surveys | Document Reference |
| --- | --- |
| Topography Survey Report | N-19084-EM-5000-V-2001 |

The Contractor must obtain all relevant engineering designs and drawings before completing the method statements and risk assessment for the works. The Contractor shall complete the table below.

| Design Reports/Drawings | Document Reference |
| --- | --- |
| PROPOSED 132kV CABLES BETWEEN BHYG G/S – YAS I S/S | N-19083.1 GG ¦ 7500 ¦ U ¦ 0001 |
| PROPOSED 132kV CABLES BETWEEN BHYG G/S – YAS I S/S Trail Pit | N-19083.1 GE ¦ ADTP ¦ N ¦ 0001 |

HSE Plan

Project Approved HSE Plan will be followed – HSE Plan No. ESPEC-HSE-D15

QA/QC Plan

- Quality Assurance Procedures - EPEC/TRANSCO_QCP_REV00 The QA procedure ensures that all construction activities for the duct bank road crossings comply with TRANSCO specifications and project requirements. - Planning & Documentation: All method statements, ITPs, material submittals, and drawings shall be submitted for approval before commencement. - Material Approvals: All materials shall be procured from TAQA-approved vendors and supported with compliance certificates. - Training & Competency: Site staff shall be trained on project QA requirements; only qualified personnel shall perform critical tasks. - Document Control: All QA documents shall be controlled, tracked, and archived by the Document Controller. - Quality Control Procedures. The QC procedure ensures that all executed works are inspected, tested, and verified to meet the specified quality standards. - Site Inspections: All stages of Route Marker & Joint Pit Marker instalaltions shall be inspected by the QC Engineer. - Testing: Visual Inspection - Inspection Requests (IRs): Submitted in advance for all HOLD/WITNESS points. Consultant approval is required to proceed. - Checklists & Logs: Standardized QA/QC checklists will be maintained daily. Records shall be available for review. - Non-Conformance Control: Any deviation shall be recorded, investigated, and corrective action shall be taken. All NCRs will be closed before final handover.

# Specifications & Standards

| TAQA Specifications & Standards | Document Reference |
| --- | --- |
| Concrete and reinforcement concrete | S-TR-CIVIL-Concrete |
| General requirements | S-TR-CIVIL-GEN |
| Civil Works for Underground Cables | S-TR-CIVIL-CAB |
| Corrosion Protection | S-TR-CORR |

| Industry Specific Standards | Document Reference |
| --- | --- |
| Quality management | ISO 9001 |

| TAQA HSE Standards | Document Reference |
| --- | --- |
| Occupational Health and Safety | ILO R164 |
| Safety and Heath in Construction | ILO R175 |
| Occupational Health, Safety Assessment Specification | OHSAS 18001 |
| System Safety Rules | HSE-HS-13 |
| Procedure for selection & Provision of PPE | HSP.31 |
| COSHH | HSP.30 |

# Permits and Permissions

| Work Permits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Permit to Dig: Required |  |  |  | Proximity to Electricity: Required (If our work is near existing electrical cables and substations) |  |  |  |
| Permit to Work (daily permit): |  |  |  | Confined Space Permit: Required (If excavation is deep) |  |  |  |
| Working under/adjacent/over utilities: Required ( if our work is involved near existing utilities) |  |  |  |  |  |  |  |
| Temporary Works Permits |  |  |  |  |  |  |  |
| Permit to Load: |  | Permit to Strike: |  |  |  |  |  |
| Permissions required to carry out the works |  |  |  |  |  |  |  |
| Tick as applicable: | Permit to Work |  | Access |  |  | Transfer |  |
| Others: N/A. |  |  |  |  |  |  |  |

| System Identification | Document Reference |
| --- | --- |
| noted & will provide necessary documents during execution of the work |  |

| Equipment/Tools | Certificate/Reference |
| --- | --- |
| Hand Shovel | N/A |
| Pick Axe | N/A |
| Excavator | N/A |
| Loaders | N/A |
| Bobcat | N/A |
| Roller Compactors | N/A |
| Plate Compactor | N/A |

The Contractor shall provide up-to-date No Objection Certificates (NOCs) from any relevant authority or utility.

| NOC | Authority/Utility |
| --- | --- |
| ADDC-TAQA Distribution | YES |
| ADNOC | YES |
| Dolphin Energy | YES |
| Du Telecom | NO |
| Etihad Rail | YES |
| Etisalat Telecom | No |
| GHQ Military Works and Telecom | No |
| ITC Right-of-Way | YES |
| MCC Monitor and Control | No |
| TWS Sewerage AD | YES |
| Tabreed Cooling | No |
| TAQA Infrastructure Services | YES |

# Specific Legal Compliance

| Activity | Document Reference |
| --- | --- |
| Transport of equipment | ILO R164 / ILO R175/HSE-HS-13 |
| Transport of filling material | ILO R164 / ILO R175/ HSE-HS-13 |
| Installation of filling material | ILO R164 / ILO R175/HSE-HS-13 |
| Compaction | ILO R164 / ILO R175/HSE-HS-13 |

| HSE Activity | Document Reference |
| --- | --- |
| Roller for compaction /Rammers / Plate Compactors | ILO R164 / ILO R175/HSE-HS-13 |
| Working with electrical tools /other equipment | ILO R164 / ILO R175/HSE-HS-13 |

# Resources & Competency

|  |  |  | Labour / Trades requirements | Numbers |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Supervisor /Competent Person |  | 1 |  |
| 2 |  | Engineer |  | 1 |  |
| 3 |  | Skilled/unskilled workers |  | 10 |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
|  |  |  | Plant and Equipment |  |  |
| 1 |  | Hiab truck |  | 1 |  |
| 2 |  | Pick Axe |  |  |  |
| 3 |  | Excavator |  |  |  |
| 4 |  | Loaders |  |  |  |
| 5 |  | Bobcat |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
|  |  |  | Materials |  |  |
| 1 |  | Route & Joint Pit Marker |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

*Note - Reference to approved mass shall be given along with material test reports.

| PPE |  |
| --- | --- |
| Mandatory: - Gloves - Hard hat - Safety footwear - High visibility vest/jacket Additional when required: - Safety glasses/goggles - Ear defenders | Specialist: |
| Additional site-specific PPE |  |

|  | Mandatory Training & Competency Requirements |  |  |
| --- | --- | --- | --- |
| Position |  | Requirements |  |
| Competent Person |  | TAQA SSR Competency Assurance Guidance Booklet No.2(SSR Summary for Contractors’ CP) Revision 1 1st January2022 HSEQ Department, TAQA |  |
| Authorized Person |  |  |  |
| HSE Engineer |  | TAQA Contractor HSE Engineer – Job Description Rev 0 |  |
| Contractor HSE Manager |  | TAQA Contractor HSE Manager – Job Description Rev 0 |  |
| Contractor HSE Officer |  | TAQA Contractor HSE Officer – Job Description Rev 0 |  |
| Site Manager / Site Engineer |  | SSR Competency Assurance Guidance Booklet No.2(SSR Summary for Contractors’ CP) Revision 11stJanuary2022HSEQ Department, TAQA |  |
| Supervisory Staff / Foreman |  | Follow TAQA requirements* |  |
| General Operative |  | Follow TAQA requirements* |  |
| Plant Operator |  | Follow TAQA requirements* |  |
| Rigger/Banksman |  | Follow TAQA requirements* |  |

*Any additional requirements to be added by TAQA as part of contractual agreements

# Communication

| Prior to commencing on works | Requirements |
| --- | --- |
| Sub-Contractor pre-start safety meeting | Always, Records required |
| On-site safety induction | Always, Records required |
| On site - competency checks prior to commencing work / QA systems | Always, Records required |

| Routine communication processes | Requirements |
| --- | --- |
| Work crews briefed on RAMS documentation | Sign off required |
| Applicable Toolbox talks (Safety / Environmental / Quality) | Sign off required |
| Contractor safety / progress meetings | Minutes of meeting |
| Workplace safety committee meetings | Minutes of meeting |
| Emergencies | Requirements |
| Emergency response actions identified and communicated | Records required |
| Accident and incident / Near Miss procedures explained | Records required |
| Task specific communication processes | Requirements |
| Follow instruction at all times from the site manager / supervisor relating to all excavation works. | Throughout duration of the works |
| Key Interfaces | Requirements |
| The Contractor shall identify key interfaces additional to the ones identified below: - TAQA - The Contractor - The Subcontractor (if any) Project meetings shall be held regularly. | Throughout duration of the works, Minutes of meeting |
| Subcontractor relationships and overall communication | Requirements |
| The Contractor shall develop a safety and overall communication plan with all Subcontractors. The details shall be provided below: A two-way daily communication shall be maintained through daily progress reports, toolbox talks, safety briefings, site visits. Etc. All the HSEQ related issues shall be discussed & communicated through weekly HSEQ review meetings & process issues shall be addressed through progress review meetings. Changes required on the method statement shall be notified immediately for the consideration of HSEQ | Where required |
| Rectification & Amendments | Requirements |
| In case of any damage, discrepancy, or change, the contractor shall bring this to the notice of the Consultant or TAQA Engineer and shall proceed further after getting necessary approval from the client. | Yes, records required |

# Supervision and Responsibilities

:

| Responsibilities | Supervisory Role |
| --- | --- |
| Responsible for execution of the contract and liaison with the Client | Site Agent / Manager |
| Responsible for supervising all teams and controls labour, plant and materials on site. | Foreman |
| Responsible for carrying out daily and process checks | Competent Person / Manager |
| Responsible for the over-all direction, monitoring, coordination, control and supply of required manpower & equipment etc. of all the construction activities and related to the defined scope of work. | Project Manager |
| Responsible for the overall implementation and monitoring of all the activities involved in defined scope of work. | Construction Manager |
| Responsible for the necessary quality assurance and control, documentation, inspections and tests as per TAQA specifications. | QA/QC Engineer |
| Directly involved in the planning and execution of the activities involved in the defined scope of work. They shall be responsible for the preparation of work plan, Safety Action Plan for work permits, if required, prior to the commencement of any work. | Site Engineer |
| Responsible for the direct execution of works involved in work processes according to the approved method statement and drawings and shall directly control and give instruction to the working personnel. He shall also ensure that works are performed with proper tools/equipment, PPE and good housekeeping around the working area. | Foreman |
| Responsible for reviewing work hazards prior to the start of the work and preparing the Safe Work Plan and work permit, if required, prior to submittal to client that must be implemented and observed during works. He must observe compliance of all safety requirements as per works procedures and HSE plan, during works | Safety Engineer |
| Daily and Process Checks |  |
| Checks to be completed (to be identified by Contractor) | Required (Y/N) |
| Method Statement Approved | Y |
| Shop Drawing Approved | Y |
| Materials used at site are approved | Y |
| Material Inspection Request approved | Y |
| Route & Joint Pit Marker locations | Y |
| Installation works complies with project requirements and approved drawings | Y |

# Health Impacts

| Health Impact | Applicable (y) | Health Impact |  | Applicable (y) |  |
| --- | --- | --- | --- | --- | --- |
| Toxicity | N | Aspiration hazard/Asthma | N |  |  |
| Skin Irritation/Dermatitis | N |  |  |  |  |
| Eye Irritation | N |  |  |  |  |
| Respiratory issues | N |  |  |  |  |
| Carcinogenetic | N |  |  |  |  |
| Reproductive toxicity | N |  |  |  |  |
| COSHH Impacts |  |  |  |  |  |
| Substances or materials |  | COSHH assessment required |  |  |  |
| Liquids -(Toxic/Corrosive/Flammable/Explosive/Environment dangerous/Irritant/Slip hazard) |  | N/A |  |  |  |
| Gases - (Toxic/Flammable/Explosive/Environment Dangerous/Irritant) |  | N/A |  |  |  |
| Dust - (Explosion/Irritant/Allergic Issues) |  | N/A |  |  |  |
| Fumes - (Toxic/Irritant/Suffocation/Explosive) |  | N/A |  |  |  |
| Vapor - (Toxic/Irritant/Suffocation) |  | N/A |  |  |  |

# Quality Control

| Name of Site Manager | Telephone Number |
| --- | --- |
| Mr. Abdul Mohsin | +971 58 962 4466 |
| Name of Competent Person | Telephone Number |
| Mr. Jayateja Podili | +971 55 380 1958 |
| Name of Contract Manager | Telephone Number |
| Mr. R Sasitharan | +971 52 851 0425 |
| Name of Person Monitoring Safety | Telephone Number |
| Mr. Mujahid Abbas | +971 54 998 4915 |
| Name of Client Contact | Telephone Number |
| Name of First Aider | Telephone Number |
| Mr Kalimuthan sathya | +971 52 986 1968 |
| Name of Fire Warden | Telephone Number |
| Mr Kalimuthan sathya | +971 52 986 1968 |
| Additional Roles |  |
| Role | Telephone Number |

| Programmed Works | Inspection/Test Activity | Specification Reference | Document Reference | Personnel | Date of Inspection/Test |
| --- | --- | --- | --- | --- | --- |
| NOC’s and permits | Documents review | TAQA STANDARD S-CAB-132-PVC-I (Rev.0-2009) 132 KV XLPE-INSULATED SINGLE-CORE UNDERGROUND CABLES | TAQA SPEC. |  |  |
| Shop Drawing Approval | Document review | TAQA STANDARD S-CAB-132-PVC-I (Rev.0-2009) 132 KV XLPE-INSULATED SINGLE-CORE UNDERGROUND CABLES | TAQA SPEC. |  |  |
| Method Statement Approval | Document review | TAQA STANDARD S-CAB-132-PVC-I (Rev.0-2009) 132 KV XLPE-INSULATED SINGLE-CORE UNDERGROUND CABLES | TAQA SPEC. |  |  |
| Survey Setting-out | Visual | TAQA STANDARD S-CAB-132-PVC-I (Rev.0-2009) 132 KV XLPE-INSULATED SINGLE-CORE UNDERGROUND CABLES | TAQA SPEC. |  |  |
| Checking for utilities | Visual | TAQA STANDARD S-CAB-132-PVC-I (Rev.0-2009) 132 KV XLPE-INSULATED SINGLE-CORE UNDERGROUND CABLES | TAQA SPEC. |  |  |
| Backfilling | Visual | TAQA STANDARD S-CAB-132-PVC-I (Rev.0-2009) 132 KV XLPE-INSULATED SINGLE-CORE UNDERGROUND CABLES | TAQA SPEC. |  |  |
| Marking locations | Visual | TAQA STANDARD S-CAB-132-PVC-I (Rev.0-2009) 132 KV XLPE-INSULATED SINGLE-CORE UNDERGROUND CABLES | TAQA SPEC. |  |  |
| Installation of Route & Joint Markers | Visual | TAQA STANDARD S-CAB-132-PVC-I (Rev.0-2009) 132 KV XLPE-INSULATED SINGLE-CORE UNDERGROUND CABLES | TAQA SPEC. |  |  |

# Environmental Impacts

| H&S References |  |  |
| --- | --- | --- |
| ILO R130-Hygiene recommendation |  |  |
| ILO R175-Safety and Health in Construction |  |  |
| HSE-HS-13-Safety Rules (TAQA) |  |  |
| COSHH HSP.30 |  |  |
| Site Access Notes and Special Arrangements: |  |  |
| Full time Coaster bus available at site with Air Conditioning in ON position Mobile Toilet Individual water bottles 20 liters cool water Thermos Fire extinguisher Fire Extinguisher |  |  |
| Site Storage Facilities |  |  |
| General Storage Requirements - All materials, equipment, and plant must be stored securely to prevent theft and damage. - Use covered or enclosed storage areas to protect materials from adverse weather conditions. Hazardous Materials - Store hazardous materials in designated areas with appropriate containment measures to prevent spills or leaks. - Clearly label all hazardous materials and their storage areas. - Keep SDS for all hazardous materials on-site and accessible to all relevant personnel. Waste Management - Establish clearly labelled waste collection bins for different types of waste (e.g., recyclables, hazardous materials, general waste). - Ensure proper storage of waste materials to prevent contamination and environmental hazards. Equipment and Tools Storage - Use lockable containers or storage units for small tools and equipment to prevent theft. |  |  |
| Waste Removal Arrangements |  |  |
| Waste Skip shall be provided as per Tadweer requirements. |  |  |
| Security Arrangements |  |  |
| - Security guards will be deployed to ensure 24/7 site security, controlling access, and preventing theft, vandalism, and unauthorized entry. - A properly secured, monitored storage facility will be provided on site for hand tools, equipment etc. |  |  |
| Management of Change at Site |  |  |
| - Management of change procedure by TAQA – Note for TAQA: To be developed by TAQA if not available. Note: - Changes can have an impact on HSE at work. These may include temporary works or works omitted from the original method statement. - The control measures for the management of change should include risk assessment, effective coordination and cooperation between all parties involved. These changes should be recorded and signed by all parties involved. |  |  |
| Special Emergency Procedures |  |  |
| As a minimum the contractor shall include the following. - Rescue plans - Emergency rescue system for working at height - Fire Emergency and Fire Prevention - Emergency Medical/First Aid response - Access and Egress plans - Environmental damage - Injuries to the employees and general public The Contractor shall evaluate the site conditions, working requirements and methodology to assess the safety emergency procedures. Where necessary they shall propose Special Emergency Procedures not covered by above or general emergency procedures. Contractor to refer to TAQA-Contractor’s camps & temporary building guidelines. |  |  |
| Environmental Impact |  |  |
| The Contractor shall work in a way as to minimize any adverse impact to the environment. They shall provide all reasonable equipment, materials and personal to minimize any risk of damage to the environment. Not limited to but as a minimum the contractor shall provide the following. - Spill control – with spill kits available on site. (Diesel, petrol or hydraulic oil) - Appropriate fuel storage - Appropriate fire prevention |  |  |

# Methodology and Sequence of Works

Employers Requirements

Pre-Requisite - All the procedures as defined on the Quality Inspection and Test Plan & Checklist shall be followed. - Prior to the commencement of the main work, necessary & relevant clearances / RFI / Approvals shall be obtained from Consultant/TAQA and other concerned authorities. - Check all environmental constraints and public constraints - Check the accessibility to the site - Check equipment and tools required for the works. - Site personnel are identified fulfilling TAQA competency requirements Construction & Installation - Security and Welfare provision are established - The working areas and sites boundaries are clearly up defined and secured - Unloading and sorting the equipment - Use barriers to keep materials personnel and equipment away from excavation edges. During darkness, the edges of an excavation must be marked with suitable lighting, especially on public thoroughfares. - Diesel or petrol engine vehicles must not exhaust into any excavation pit, exhaust ventilation may need to be provided, this may also constitute a confined space - Top surface backfill shall consist of normal site surface soil

Sequence of Work

- Prior Activities - Sequence of Activities

Method Statement

This methodology prepared as per the TAQA specification TAQA STANDARD : TAQA STANDARD S-CAB-132-PVC-I (REV.0-2019) Prior to commencement of excavation works; - Method Statement must be approved and will be available at site - Clearance and Permit to Works must be obtained. - Methodology for the execution of works shall be explained to all involved personnel and approved method statement shall be made available at site. - All plant & equipment and personnel certification will be submitted to site O/E for verification before start of work. - Appropriate Personal Protective Equipment, “PPE” (Hand gloves, Ear plugs/Earmuffs, Safety glasses, Safety vest, Hard hats and safety shoes) shall be used. - Toolbox meeting shall be conducted daily prior to site activities. - Obtain required permits / NOCs from the concerned authorities. - Excavation backfilling works shall be commenced after getting the Activity Commence Request Approval from the concerned Utility In-Charge / Engineer and works shall be carried out by Competent person approved by the relevant utility owners such as TAQA etc. - HSE documents shall be approved prior to commencement of activities. - Display traffic signboards, surround the working area by safety barriers (plastic/concrete), safety cones, and safety tape and install flash lights on roadside before commencing the excavation as per the local authority requirements. - LAP will be available at site - LAP will be taken wherever required. Approved CP by TAQA/ADDC will be available on site all time during work, controlling all activities & persons. - Ensure availability of approved materials. 8.1 Sequence of Activities -Backfilling - For the details of backfilling carried out, please refer to Method Statement for backfilling submitted (Add document No.) - Once the Backfilling & Reinstatement is completed the following steps will be followed Route Marker & Joint Pit Marker Installation: - Route Marker location will be marked as per approved route marker location drawing, along the cable route. - Manual excavation to be carried out as per approved corridor/profile & alignment - Excavated Cable route centre coordinates & levels to be verified by Consultant. - Place route markers, projecting 15 cm above ground. at the specified intervals along the cable route as per the specifications. - Crash Poles/Joint Marker shall be erected at the four outside corners of the Joint Pit - The area to be backfilled immediately after installation of the route markers. - The final level shall be watered & compacted as per requirements. - Surveyor will record the GPS coordinates of every route marker, Joint markers, Chainage marking, offset from the nearest permanent structure for As built requirements and submitted to Consultant and TAQA Apporval. - Cable route should be clean & tidy and ready for initiating final inspection & Handing over.

Verification/Validation pause points

- Route Marker & Joint Pit Marker – level and location be approved by TAQA/TAQA’s consultant - Marker location verification - Marker fixing/alignment - Backfilling & reinstatement - Final survey

Lifting plan arrangement

Not Applicable for the activity

| Testing and Commissioning – Not Applicable – No HV/LV Equipment |  |
| --- | --- |
| TAQA Approved Personnel | Standard Test Equipment ID No. |

# Emergency Procedures

| Hospital (A & E) |  |
| --- | --- |
| NMC Royal Hospital-Khalifa City | 8001122 |
| Burjeel Medical Centre Al Zeina-Al Raha | 025506691 |
| Yas Marina Circuit Medical Centre | 026599800 |
| Emergency Contact Numbers |  |
| Emergency / Police / Fire / Ambulance | Police-999, Fire- 997, Ambulance-998 |
| First Aid Appointed Persons on site | Mr Kalimuthan Sathya |
| First Aid Equipment | First Aid Box |
| Marine Emergencies | 996 |
| In case of damage to any of the services listed below, use the numbers listed to report the damage: |  |
| TAQA | 024164207 |
| Electricity | 991 |
| Water | 992 |

# Appendices

Document/Drawings/Reports/etc.

# Risk Assessments

|  | Risk Assessments |  |
| --- | --- | --- |
| - Risk Assessment - |  |  |

| Prepared by: Site Engineer/Competent Person | Reviewed by: EHS Engineer/ Site Manager | Approved by: Project Manager/Site Manager | Consultant Approval |
| --- | --- | --- | --- |

# Sign Off Sheet

| Name | Signature | Company | Date |
| --- | --- | --- | --- |