MSRA FOR WATERPROOFING WORKS

Document Reference Number: EM-5000-M-2010-RA

Location: YASI S/S to BHYG S/S

Work Scope: Waterproofing Works

Client Contract No: N 19084

Contractor is responsible to prepare method statement for each work activities with compliance to RFC drawings, standard drawings, project specification, prevailing international standards and other relevant approvals and submit it for consultant/TAQA review and approval.

This method statement guideline is prepared in good faith in line with standard engineering practices, to guide, facilitate contractors and standardize the procedural submissions. However, it does not relieve the Contractor from their contractual obligations.

Any discrepancy between this information and actual site conditions shall be verified for its correctness and suitability, as per applicable TAQA procedures and local authorities. The Contractor shall verify and confirm submissions case to case, as per applicable law and standards.

| DOCUMENT REVIEW & AUTHORISATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor |  | APPROVAL |  |  |  |  |  |
|  |  | Contractor Project Manager | Sub-Contractor Project Manager | Site Manager | Duty Holder/ HSE Manager | Client/Client Representative |  |
| Name |  | Mr. R Sasitharan | Yousuf Suleiman |  |  |  |  |
| Signature /Date |  |  |  |  |  |  |  |
| 1 | Does this document identify high risk activities? Yes / No If answer is “Yes” then risk assessment for each individual high-risk element of work shall be included. |  |  |  |  |  |  |
| 2 | Does the work scope include temporary works? Yes / No If answer to above question is “Yes” then a separate risk assessment/method statement shall be submitted for temporary works in addition to the method statement risk assessment for the main works |  |  |  |  |  |  |

|  | DISTRIBUTION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ORGANISATION NAME |  | NAME | POSITION | SIGNATURE | DATE |  |

Sub-Contractor  EPC Contractor  (OE) PM Consultant appointed by the Client  Client

|  | REVISION RECORD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Revision |  |  | Date |  |  | Comments / Changes |  |
| 0 |  |  | 30/10/2025 |  |  | First issue |  |  |
|  | Field Change Notices |  |  |  |  |  |  |  |

Contents

Contents

Abbreviations Used	3

1. Purpose and Scope of Works	4

2. Programme and Constraints	5

3. Surveys and Engineering Design	6

4. Specifications & Standards	8

5. Permits and Permissions	9

6. Specific Legal Compliance	10

7. Resources & Competency	11

8. Communication	13

9. Supervision and Responsibilities	14

10. Health Impacts	15

11. Quality Control	16

12. Environmental Impacts	18

13. Methodology and Sequence of Works	20

14. Emergency Procedures	23

15. Appendices	24

16. Risk Assessments	25

17. Sign Off Sheet	26

# Abbreviations Used

| Abbreviations Used | Definition |
| --- | --- |
| ILO R164 | Occupational Safety and Heath Recommendation |
| ILO R175 | Safety and Heath in Construction |
| PPE | Personal protective equipment |
| COSHH | Control of Substances Hazardous to Health |

# Purpose and Scope of Works

Purpose

To define the methodology of waterproofing works across YASI S/S to BHYG S/S Areas for cast in situ installations and to ensure that the works are executed safely and to the contract quality requirements.

Scope

The scope of work includes waterproofing works for 132kV Cable installation cast in situ installations for this project in compliance with the contract specification and required standards

# Programme and Constraints

The working hours will be in accordance with the contractual agreements for the project, governments guidelines and restrictions imposed by the local authorities.

| Duration of Works |  |  |  |  |
| --- | --- | --- | --- | --- |
| Anticipated start date |  | Anticipated finish date: |  | Duration: 3 Months |
| Working Hours |  |  |  |  |
| Weekday | Start time: 7:00 AM |  | Finish time: 5:00 PM |  |
| Weekend | Start time: |  | Finish time: |  |
| Generic Constraints |  |  |  |  |
| Environmental: Spilling of fuel or hydraulic oil on site |  |  |  |  |
| Equipment/Plant/Machinery: Excavator |  |  |  |  |
| Site Specific Constraints |  |  |  |  |
| Environmental: Identify Potential Sources of Waste: Wood, paper. Waste Segregation and Storage: Establish clearly labelled waste collection bins for different types of waste, such as recyclables, hazardous materials, and general waste.Ensure proper storage of hazardous materials in designated areas with appropriate containment measures to prevent spills or leaks. Waste Disposal and Recycling: Identify local regulations and guidelines for the disposal of different types of waste, including hazardous materials. Contract with licensed waste management companies for the proper disposal of hazardous materials and recycling of recyclable materials. Implement procedures for the safe transportation of hazardous materials to disposal facilities. Monitoring and Evaluation: Regularly monitor waste generation and disposal practices to identify areas for improvement. Conduct periodic audits to ensure compliance with waste management procedures and regulations. Dust can be produced in dry condition; as a result dust mask may be required. If required, water spray will be used to suppress dust. |  |  |  |  |
| Equipment/Plant/Machinery: Excavator |  |  |  |  |
| Existing Infrastructure: The presence of any existing structures or underground elements |  |  |  |  |
| Utilities: - Obtaining approval from utility authority and relevant department’s prior commencement of work. - Obtaining existing utilities information and excavate trial pits to locate and confirm the existing services at trench route as necessary. - Check Calibration of Surveying Equipment’s. - Check for existing utilities before excavation. |  |  |  |  |
| Access/Egress etc.: - Only operatives inducted with valid third-party training certificates will be allowed on site. - The site will have designated walkways/walking routes for pedestrians and roads for vehicles, with a speed limit of 20 km/hr. - Parking will be available on site for personnel. - The work area will be clearly demarcated to restrict access and mitigate unauthorized entry. |  |  |  |  |

# Surveys and Engineering Design

If the works are within existing infrastructure, then procure any historical or current as built records of the area from the relevant parties, including third party. Necessary data survey support from site in order to identify the risk implication based on actual site conditions such as underground services will be given

The following factors can influence the selection of construction method and equipment:

- Accessibility to the location

- Ground conditions

- Existing Infrastructure

- Proximity to live equipment

| Surveys | Document Reference |
| --- | --- |
| Topography Survey | In progress |

| Design Reports/Drawings | Document Reference |
| --- | --- |
| PROPOSED 132kV CABLES BETWEEN BHYG G/S – YAS I S/S | N-19083.1 GG ¦ 7500 ¦ U ¦ 0001 |
| PROPOSED 132kV CABLES BETWEEN BHYG G/S – YAS I S/S Trail Pit | N-19083.1 GE ¦ ADTP ¦ N ¦ 0001 |

A project HSE plan will be provided in accordance with the mandatory requirements of TAQA T&D 12 – Risk Management Procedure.

HSE Plan

Project Approved HSE Plan will be followed – HSE Plan No. ESPEC-HSE-D15

A project Quality Assurance/Quality Control plan will be provided in accordance with the mandatory requirements of TAQA T&D 12 – Risk Management Procedure.

QA/QC Plan

- Quality Assurance Procedures - EPEC/TRANSCO_QCP_REV00 The QA procedure ensures that all construction activities for the duct bank road crossings comply with TRANSCO specifications and project requirements. - Planning & Documentation: All method statements, ITPs, material submittals, and drawings shall be submitted for approval before commencement. - Material Approvals: All materials shall be procured from TAQA-approved vendors and supported with compliance certificates. - Training & Competency: Site staff shall be trained on project QA requirements; only qualified personnel shall perform critical tasks. - Document Control: All QA documents shall be controlled, tracked, and archived by the Document Controller. - Quality Control Procedures. The QC procedure ensures that all executed works are inspected, tested, and verified to meet the specified quality standards. - Site Inspections: All stages of duct bank construction (excavation, duct laying, concreting, waterproofing, backfilling, reinstatement) shall be inspected by the QC Engineer. - Testing: Concrete, backfill, and TR materials shall be tested as per project ITPs. Mandrel testing shall be done prior to cable pulling. - Inspection Requests (IRs): Submitted in advance for all HOLD/WITNESS points. Consultant approval is required to proceed. - Checklists & Logs: Standardized QA/QC checklists will be maintained daily. Records shall be available for review. - Non-Conformance Control: Any deviation shall be recorded, investigated, and corrective action shall be taken. All NCRs will be closed before final handover.

# Specifications & Standards

| TAQA Specifications & Standards | Document Reference |
| --- | --- |
| Metal works | S-TR-CIVIL-METAL |
| General requirements | S-TR-CIVIL-GEN |
| Substations and Buildings | S-TR-CIVIL-BUILD |

| Industry Specific Standards | Document Reference |
| --- | --- |
| Quality management | ISO 9001 |

| TAQA HSE Standards | Document Reference |
| --- | --- |
| Occupational Health and Safety | ILO R164 |
| Safety and Heath in Construction | ILO R175 |
| Occupational Health, Safety Assessment Specification | OHSAS 18001 |
| System Safety Rules | HSE-HS-13 |
| Procedure for selection & Provision of PPE | HSP.31 |
| COSHH | HSP.30 |

# Permits and Permissions

All the work permits will be identified  that will be required to undertake delivery of the scope of works defined in Section 1.

Identify any system clearance requirements and obtain permits from the system operator, as required.

| Work Permits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Permit to Dig: YES |  |  |  | Proximity to Electricity: Required (If our work is near existing electrical cables and substations) |  |  |  |
| Permit to Work (daily permit): |  |  |  | Confined Space Permit: YES |  |  |  |
| Working under/adjacent/over utilities: Required ( if our work is involved near existing utilities) |  |  |  |  |  |  |  |
| Temporary Works Permits |  |  |  |  |  |  |  |
| Permit to Load: |  | Permit to Strike: |  |  |  |  |  |
| Permissions required to carry out the works |  |  |  |  |  |  |  |
| Tick as applicable: | Permit to Work |  | Access |  |  | Transfer |  |
| Others: N/A. |  |  |  |  |  |  |  |

| System Identification | Document Reference |
| --- | --- |
| noted & will provide necessary documents during execution of the work |  |

The Contractor shall provide up-to-date testing/fitness/calibration certificates for proposed tools and equipment.

| Equipment/Tools | Certificate/Reference |
| --- | --- |

.

| SN | Authority/Utility | PTW required |
| --- | --- | --- |
| 1 | ADDC-TAQA Distribution | YES |
| 2 | ADNOC | YES |
| 3 | Dolphin Energy | YES |
| 4 | Du Telecom | NO |
| 5 | Etihad Rail | YES |
| 6 | Etisalat Telecom | No |
| 7 | GHQ Military Works and Telecom | No |
| 8 | ITC Right-of-Way | YES |
| 9 | MCC Monitor and Control | No |
| 10 | TWS Sewerage AD | YES |
| 11 | Tabreed Cooling | No |

# Specific Legal Compliance

| Activity | Document Reference |
| --- | --- |
| Transport of equipment and materials | ILO R164 / ILO R175/HSE-HS-13 |
| Installation of adhesive/insulation/bitumen | ILO R164 / ILO R175/ HSE-HS-13 |

| HSE Activity | Document Reference |
| --- | --- |
| Working with chemicals | ILO R164 / ILO R175/HSE-HS-13 |
| Lifting operation | ILO R164 / ILO R175/HSE-HS-13 |
| Manual handling/sharp edges | ILO R164 / ILO R175/HSE-HS-13 |
| Working at height | ILO R164 / ILO R175/HSE-HS-13 |

# Resources & Competency

|  |  |  | Labour / Trades requirements | Numbers |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Supervisor/Engineer |  | 1 |  |
| 2 |  | Skilled/Unskilled works |  | 5 |  |
| 3 |  | Technicaian |  | 1 |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
|  |  |  | Plant and Equipment |  |  |
| 1 |  | Truck |  | 1 |  |
| 2 |  | Wire Brush |  |  |  |
| 3 |  | Trowels |  |  |  |
| 4 |  | Hand Roller |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
|  |  |  | Materials |  |  |
| 1 |  | Waterproofing Membrane |  |  |  |
| 2 |  | Adhesives |  |  |  |
| 3 |  | Bitumen |  |  |  |
| 4 |  |  |  |  |  |

| PPE |  |
| --- | --- |
| Mandatory: - Gloves - Safety Helmet - Safety footwear - High visibility vest/jacket Additional when required: - Safety glasses/goggles - Ear defenders - Welding mask - Respiratory protection | Specialist: - Full body Harnesses with double lanyard - Fall arrest system - Fall Restraint |
| Additional site-specific PPE |  |

|  | Mandatory Training & Competency Requirements |  |  |
| --- | --- | --- | --- |
| Position |  | Requirements |  |
| Competent Person |  | TAQA SSR Competency Assurance Guidance Booklet No.2(SSR Summary for Contractors’ CP) Revision 1 1st January2022 HSEQ Department, TAQA |  |
| Authorized Person |  |  |  |
| HSE Engineer |  | TAQA Contractor HSE Engineer – Job Description Rev 0 |  |
| Contractor HSE Manager |  | TAQA Contractor HSE Manager – Job Description Rev 0 |  |
| Contractor HSE Officer |  | TAQA Contractor HSE Officer – Job Description Rev 0 |  |
| Site Manager / Site Engineer |  | SSR Competency Assurance Guidance Booklet No.2(SSR Summary for Contractors’ CP) Revision 11stJanuary2022HSEQ Department, TAQA |  |
| Supervisory Staff / Foreman |  | Follow TAQA requirements* |  |
| General Operative |  | Follow TAQA requirements* |  |
| Plant Operator |  | Follow TAQA requirements* |  |

# Communication

| Prior to commencing on works | Requirements |
| --- | --- |
| Sub-Contractor pre-start safety meeting | Always, Records required |
| On-site safety induction | Always, Records required |
| On site - competency checks prior to commencing work / QA systems | Always, Records required |

| Routine communication processes | Requirements |
| --- | --- |
| Work crews briefed on RAMS documentation | Sign off required |
| Applicable Toolbox talks (Safety / Environmental / Quality) | Sign off required |
| Contractor safety / progress meetings | Minutes of meeting |
| Workplace safety committee meetings | Minutes of meeting |
| Emergencies | Requirements |
| Emergency response actions identified and communicated | Records required |
| Accident and incident / Near Miss procedures explained | Records required |
| Task specific communication processes | Requirements |
| Follow instruction at all times from the site manager / supervisor relating to all excavation works. | Throughout duration of the works |
| Key Interfaces | Requirements |
| The Contractor shall identify key interfaces additional to the ones identified below: - TAQA - The Contractor - The Subcontractor (if any) Project meetings shall be held regularly. | Throughout duration of the works, Minutes of meeting |
| Subcontractor relationships and overall communication | Requirements |
| The Contractor shall develop a safety and overall communication plan with all Subcontractors. The details shall be provided below: | Where required |
| Rectification & Amendments | Requirements |
| In case of any damage, discrepancy, or change, the contractor shall bring this to the notice of the Consultant or TAQA Engineer and shall proceed further after getting necessary approval from the client. | Yes, records required |

# Supervision and Responsibilities

| Responsibilities | Supervisory Role |
| --- | --- |
| Responsible for execution of the contract and liaison with the Client | Site Agent / Manager |
| Responsible for supervising all teams and controls labour, plant and materials on site. | Foreman |
| Responsible for carrying out daily and process checks | Competent Person / Manager |
| Responsible for the over-all direction, monitoring, coordination, control and supply of required manpower & equipment etc. of all the construction activities and related to the defined scope of work. | Project Manager |
| Responsible for the overall implementation and monitoring of all the activities involved in defined scope of work. | Construction Manager |
| Responsible for the necessary quality assurance and control, documentation, inspections and tests as per TAQA specifications. | QA/QC Engineer |
| Directly involved in the planning and execution of the activities involved in the defined scope of work. They shall be responsible for the preparation of work plan, Safety Action Plan for work permits, if required, prior to the commencement of any work. | Site Engineer |
| Responsible for the direct execution of works involved in work processes according to the approved method statement and drawings and shall directly control and give instruction to the working personnel. He shall also ensure that works are performed with proper tools/equipment, PPE and good housekeeping around the working area. | Foreman |
| Responsible for reviewing work hazards prior to the start of the work and preparing the Safe Work Plan and work permit, if required, prior to submittal to client that must be implemented and observed during works. He must observe compliance of all safety requirements as per works procedures and HSE plan, during works | Safety Engineer |
| Daily and Process Checks |  |
| Checks to be completed | Required (Y/N) |
| Method Statement Approved | Y |
| Shop Drawing Approved | Y |
| ITP approved | Y |
| Materials used at site are approved | Y |
| Material Inspection Request approved | Y |
| Surveying | Y |
| Blinding Works |  |
| Surface Preparation | Y |
| Application of Primer | Y |
| Application of Membrane | Y |
| Surface Works for Duct |  |
| Surface Preparation | Y |
| Application of Primer | Y |
| Application of Membrane | Y |
| Mastic Application | Y |

# Health Impacts

| Health Impact | Applicable (y) | Health Impact |  | Applicable (y) |  |
| --- | --- | --- | --- | --- | --- |
| Toxicity | Y | Aspiration hazard/Asthma | Y |  |  |
| Skin Irritation/Dermatitis | Y |  |  |  |  |
| Eye Irritation | Y |  |  |  |  |
| Respiratory issues | Y |  |  |  |  |
| Carcinogenetic | Y |  |  |  |  |
| Reproductive toxicity | Y |  |  |  |  |
| COSHH Impacts |  |  |  |  |  |
| Substances or materials |  | COSHH assessment required |  |  |  |
| Liquids -(Toxic/Corrosive/Flammable/Explosive/Environment dangerous/Irritant/Slip hazard) |  | YES |  |  |  |
| Gases - (Toxic/Flammable/Explosive/Environment Dangerous/Irritant) |  | YES |  |  |  |
| Dust - (Explosion/Irritant/Allergic Issues) |  | YES |  |  |  |
| Fumes - (Toxic/Irritant/Suffocation/Explosive) |  | YES |  |  |  |
| Vapor - (Toxic/Irritant/Suffocation) |  | YES |  |  |  |

# Quality Control

| Name of Site Manager | Telephone Number |
| --- | --- |
| Mr. Abdul Mohsin | +971 58 962 4466 |
| Name of Competent Person | Telephone Number |
| Mr. Jayateja Podili | +971 55 380 1958 |
| Name of Contract Manager | Telephone Number |
| Mr. R Sasitharan | +971 52 851 0425 |
| Name of Person Monitoring Safety | Telephone Number |
| Mr. Mujahid Abbas | +971 54 998 4915 |
| Name of Client Contact | Telephone Number |
| Name of First Aider | Telephone Number |
| Mr Kalimuthan sathya | +971 52 986 1968 |
| Name of Fire Warden | Telephone Number |
| Mr Kalimuthan sathya | +971 52 986 1968 |
| Additional Roles |  |
| Role | Telephone Number |

ITP will be as follows:

| Programmed Works | Inspection/Test Activity | Specification Reference | Document Reference | Personnel | Date of Inspection/Test |
| --- | --- | --- | --- | --- | --- |
| Shop drawing Approval | Doc.Review | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Method statement Approval | Doc.Review | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Material receiving at site | Doc.Review | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Material Storage | Visual | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Blinding Waterproofing | Visual, Work inspection request | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Surface Preparation | Visual | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Application of Primer | Visual | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Application of Membrane | visual | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Surface Waterproofing | Visual, Work inspection request | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Surface Preparation | Visual | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Application of Primer | Visual | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |
| Application of Membrane | visual | S-TR-CIV-CONCRETE (REV.0-2015) | TAQA Spec. |  |  |

# Environmental Impacts

| H&S References |  |  |
| --- | --- | --- |
| ILO R130-Hygiene recommendation |  |  |
| ILO R175-Safety and Health in Construction |  |  |
| HSE-HS-13-Safety Rules (TAQA) |  |  |
| COSHH HSP.30 |  |  |
| Site Access Notes and Special Arrangements: |  |  |
| Welfare facilities will be provided and details of these will be agreed with TAQA before production of this method statement. Details of the agreed Welfare Facilities will be included in this section. TAQA-Contractor’s camps & temporary building guidelines will be referred - Full time Coaster bus available at site with Air Conditioning in ON position - Mobile Toilet - Individual water bottles - 20 liters cool water Thermos - Fire extinguisher - First aid box and trained first aider |  |  |
| Site Storage Facilities |  |  |
| General Storage Requirements - All materials, equipment, and plant must be stored securely to prevent theft and damage. - Use covered or enclosed storage areas to protect materials from adverse weather conditions. Hazardous Materials - Store hazardous materials in designated areas with appropriate containment measures to prevent spills or leaks. - Clearly label all hazardous materials and their storage areas. - Keep SDS for all hazardous materials on-site and accessible to all relevant personnel. Waste Management - Establish clearly labelled waste collection bins for different types of waste (e.g., recyclables, hazardous materials, general waste). - Ensure proper storage of waste materials to prevent contamination and environmental hazards. Equipment and Tools Storage - Use lockable containers or storage units for small tools and equipment to prevent theft. |  |  |
| Waste Removal Arrangements |  |  |
| Waste Skip shall be provided as per Tadweer requirements. |  |  |
| Security Arrangements |  |  |
| - Security guards will be deployed to ensure 24/7 site security, controlling access, and preventing theft, vandalism, and unauthorized entry. - A properly secured, monitored storage facility will be provided on site for hand tools, equipment etc. |  |  |
| Management of Change at Site |  |  |
| - Management of change procedure by TAQA – Note for TAQA: To be developed by TAQA if not available. Note: - Changes can have an impact on HSE at work. These may include temporary works or works omitted from the original method statement. The control measures for the management of change should include risk assessment, effective coordination and cooperation between all parties involved. These changes should be recoded and signed by all parts involved. |  |  |
| Special Emergency Procedures |  |  |
| - Emergency Medical/First Aid response - Access and Egress plans - Environmental damage - Injuries to the employees and general public - Contractor to refer to TAQA-Contractor’s camps & temporary building guidelines. |  |  |
| Environmental Impact |  |  |
| - Spill control – with spill kits available on site. (Diesel, petrol or hydraulic oil) - Appropriate fuel storage - Appropriate fire prevention |  |  |

# Methodology and Sequence of Works

Employers Requirements

Pre-Requisite - Availability of approved RFC and other relevant drawings. - All required approved materials as per engineering drawings. - All the procedures as defined on the Quality Inspection and Test Plan & Checklist shall be followed. - Materials must be shifted to the working area at site using suitable equipment and must be handled with care to avoid any damage. - Check all environmental constraints and public constraints - Check equipment and tools required for the works. Ensure that all equipment’ certificates are available and within the validity period (inspection and maintenance records) - All available options for construction techniques and equipment with respect to the specific site conditions are reviewed and evaluated. - Site personnel are identified fulfilling TAQA competency requirements Construction & Installation Installation - Security and Welfare provision are established - The working areas and sites boundaries are clearly up defined and secured - All works must be executed by skilled workers - Surface preparation-use of solvents and chemicals - Sealing - Caulking - Wind tightening - Insulation of roof - Installation of waterproofing membranes / Bituminous material/OTHER

Sequence of Work – To be filled by the contractor

- Prior Activities - Sequence Of Activities

Method Statement

- Prior Activities ### Orientation to be done for all staff regarding safety and environmental requirements before sending to site ### Personal Protective Equipment (PPEs) must be available for all personnel /staff. ### Obtaining approval from utility authority and relevant authority / department prior to commencement of work. ### Obtaining existing utilities information and excavate trial pits to locate and confirm the existing services at trench route and obtain verification for the same as necessary. ### Obtaining the approval of shop drawing, route approval, materials and method statement prior to start of activity ### A copy of all the approvals of all the drawings & all the materials, shall be available on site all time. ### Check all related shop drawings are approved and available at site ### Check physically for existing utilities before excavation ### Survey & mark the points for Duct bank & Cast In Situ installation as per Engineer approved Shop drawing ### Excavate the cable trench for Duct bank construction as per approved cable route drawing - GPR and voltage detectors will be used prior to excavation - Excavation for road crossings will be carried out in line with the instructions of concerned authorities, targeting to leave at least half of the road open to the traffic at all times ## Sequence Of Works Waterproofing for Blinding - Surface Preparation: The concrete surface shall be cleaned thoroughly of all contaminants like dust, traces of curing compound, oil, grease. - All surface imperfections and protrusions are to be removed and repaired. - All concrete surfaces shall be primed by solvent based bitumen primer. Mix the contents of the pail/drum prior to the application to remove any sediment. Apply the primer @ 4-6 m²/lt by a brush, roller or an airless spray. Allow the primer to dry prior to the application of the waterproof membrane. - Waterproof membranes shall be installed from the low point or drains, so the flow of water is over or parallel to the plies, but never against the laps. All overlaps at the membrane seams shall be installed so as to have "up" slope laps over "down" slope laps. - The applied membranes shall be protected immediately from the ongoing site activities or from sharp aggregates during backfilling with a tough, weather, warp and rot proof asphaltic protection board of suitable thickness. Waterproofing after casting - Surface Preparation: The concrete surface shall be cleaned thoroughly of all contaminants like dust, traces of curing compound, oil, grease. - All surface imperfections and protrusions are to be removed and repaired. - All concrete surfaces shall be primed by solvent based bitumen primer. Mix the contents of the pail/drum prior to the application to remove any sediment. Apply the primer @ 4-6 m²/lt by a brush, roller or an airless spray. Allow the primer to dry prior to the application of the waterproof membrane. - Waterproof membranes shall be installed from the low point or drains, so the flow of water is over or parallel to the plies, but never against the laps. All overlaps at the membrane seams shall be installed so as to have "up" slope laps over "down" slope laps. - The applied membranes shall be protected immediately from the ongoing site activities or from sharp aggregates during backfilling with a tough, weather, warp and rot proof asphaltic protection board of suitable thickness. - Fixing by Synthetic Rubber Adhesive: Peel off the anti-stick PE film from the underside of the protection board. Apply the adhesive on various spots on the protection board. Press the board immediately onto the substrate. Provide supports/props to keep the boards in place till the adhesive is strong enough to hold the board. - Apply sealant either with a caulking gun or by a spatula. Ensure that the mastic is firmly pressed into the joint. Avoid any air pockets. Tool the mastic with suitable tooling equipment soaked in clean water. Immediately after the application of the mastic, remove the masking tape from the joint edges to give a neat finish look.

Verification/Validation pause points

- Water proofing test – To be witnessed by TAQA/TAQA’s Consultant

Lifting plan arrangement

Not Applicable for this activity

| Testing and Commissioning – Not Applicable – No HV/LV Equipment |  |
| --- | --- |
| TAQA Approved Personnel | Standard Test Equipment ID No. |

# Emergency Procedures

| Hospital (A & E) |  |
| --- | --- |
| NMC Royal Hospital-Khalifa City | 8001122 |
| Burjeel Medical Centre Al Zeina-Al Raha | 025506691 |
| Yas Marina Circuit Medical Centre | 026599800 |
| Emergency Contact Numbers |  |
| Emergency / Police / Fire / Ambulance | Police-999, Fire- 997, Ambulance-998 |
| First Aid Appointed Persons on site | Mr Kalimuthan Sathya |
| First Aid Equipment | First Aid Box |
| Marine Emergencies | 996 |
| In case of damage to any of the services listed below, use the numbers listed to report the damage: |  |
| TAQA | 024164207 |
| Electricity | 991 |
| Water | 992 |

# Appendices

Document/Drawings/Reports/etc.

# Risk Assessments

|  | Risk Assessments |  |
| --- | --- | --- |
| Annexure 1 : Risk Assessment |  |  |

| Prepared by: Site Engineer/Competent Person | Reviewed by: EHS Engineer/ Site Manager | Approved by: Project Manager/Site Manager | Consultant Approval |
| --- | --- | --- | --- |
| Kevin Anto |  |  |  |

# Sign Off Sheet